Condition Report for JS330LC

Compiled by Stuart Summers for TuffWerx
Summary:

The excavator appears to be in excellent condition.  There is some cosmetic damage in the form of faded decals, scratches and rust. However, none looks to be impacting the functionality of the equipment.  Pat Strutz from Prestige Equipment started the engine and engaged all functions of the excavator. It appeared to be working fine but a hydraulic leak was discovered during this test. Pat described it as a leak at a clamp. I took a picture of this area but it was hard to see in far enough to get a good view of it.

Details:

Paint/Exterior: generally the paint and exterior look clean and in good condition.  There are many areas of scratches on the back panel, with the largest area being the bottom of both sides. The scratches along the bottom corners are rusting. 

[image: image1.jpg]



There is a dented scratch at the top right corner of the rear left panel. It did not penetrate the paint though.

[image: image2.jpg]



The Boom and Arm are in excellent condition with few scratches to paint except near the bucket. The section of the arm directly above the bucket has scratches through the paint.

[image: image3.jpg]



Hydraulic lines on Boom appear to be in excellent condition.

There is some lubricant leaking from the up/down pivot points of the boom. However this seems like it may be normal.

[image: image4.jpg]


   [image: image5.jpg]



The tracks look normally worn. I do not see any damage.

[image: image6.jpg]


  [image: image7.jpg]


 

Engine:

The engine looks very clean. There is a lot of rust on the muffler, although this seems normal.

[image: image8.jpg]



I could see three filters, and all were replaced on 11/08/07 at 2414 hours. The engine currently has 3280 hrs. Pat said the engine is in need of servicing.

[image: image9.jpg]


 [image: image10.jpg]



There was one filter casing that was cracked and had been repaired. This was not connected to anything and Pat said it was an old filter assembly that had been replaced with another filter.

[image: image11.jpg]



Interior:

The interior is clean. The seat is in good condition and the controls all seem to be in good condition.

[image: image12.jpg]


 [image: image13.jpg]


 

Engine starting and function:

When Pat started the engine, it started on the first try and smoked briefly before settling into a steady idle. He demonstrated that both transits worked at the same speed.

Pat did discover a leak in a clamp in one of the hydraulic hoses. He intends to have this fixed.
